# VISVESVARAYA TECHNOLOGICAL UNIVERSITY BELAGAVI-590018

Project Phase – I (22ISP65)

Report on

“Design and Development of a Flexible AI-Integrated Intraoral

Imaging Device for Real-Time Dental Diagnostics”

Submitted in the partial fulfillment of the requirement for the Project

Phase - I

## Bachelor of Engineering in

Information Science and Engineering

Submitted by

1. Mr. Chethan K P      1NC22IS010

2. Mr. Dhanraj M V     1NC22IS012

3. Mr. Keerthi M          1NC22IS026

4. Mr. Ullas N G           1NC22IS060

Under the Guidance of

Dr. Ramesh M. Kagalkar Professor Dept.of ISE

Department of Information Science and Engineering

### NAGARJUNA COLLEGE OF ENGINEERING AND TECHNOLOGY

(An Autonomous Institution Approved by AICTE, affiliated to VTU, Belagavi-590018)

Venkatagiri Kote, Devanahalli, Bengaluru-562110

## 2024-2025

NAGARJUNA COLLEGE OF ENGINEERING AND TECHNOLOGY

(An Autonomous Institution Approved by AICTE, affiliated to VTU, Belagavi-590018)

#### Venkatagiri Kote, Devanahalli, Bengaluru-562110

# CERTIFICATE

#### This is to certify that the project work entitled “AI-IoT Smart Tree Guardian: A Solar-Powered Intelligent System for Tree Health Monitoring, Protection, and Market Valuation” is carried out by Miss. Gaganashree A. (1NC22IS015) , Miss. N Valli Gayathri (1NC22IS037) , Miss. Prakruthi P.H (1NC22IS041), Miss. Shree Vidhya R.( 1NC22IS054) bonafide student of VIth semester B.E (ISE), Nagarjuna College of Engineering and Technology, Bengaluru, in partial fulfillment for the award of Bachelor of Engineering in Information Science and Engineering department of the Visvesvaraya Technological University (VTU), Belagavi during the year 2024-2025. It is certified that all corrections/suggestions indicated for Internal Assessment have been incorporated in the report. The project report has been approved as it satisfies the academic requirements concerning the Project Phase – I (22ISP65) prescribed for the said degree.

Signature of the Guide	Signature of the HOD	Signature of the Principal

Dr. Ramesh M. Kagalkar Professor Dept.of ISE

Dr.	Sanjeevakumar	M.	Hatture Professor & HOD

Dr. B. V. Ravishankar Principal

ACKNOWLEDGMENT

It is our proud privilege and duty to acknowledge the kind of help and guidance received from several people in preparation for this Project Phase - I report. It would not have been possible to prepare this report, in this form without their valuable help, cooperation, and guidance. We would like to thank NGI Management for the constant support and facilities provided in the carrying out of the Project Phase - I on college premises.

We wish to record my sincere gratitude to Dr. B. V. Ravishankar, Principal, Nagarjuna College of Engineering and Technology for his constant support and encouragement in the preparation of this project and for providing the library and laboratory facilities needed to prepare this Project Phase – I report.

We extend my heartfelt gratitude to Dr. Mallikarjun M. Kodabagi, Professor and Dean of the School of Computing, as well as Head of the Department of Computer Science and Engineering at Nagarjuna College of Engineering and Technology, for his invaluable support, cooperation, insightful suggestions.

We would like to thank Project Phase – I Coordinator Dr. Sanjeevakumar M Hatture, Professor and Head, Department of Information Science and Engineering, Nagarjuna College of Engineering and Technology for his valuable suggestions and guidance throughout the preparation of the Project Phase – I report. His expertise and encouragement have been instrumental in shaping the successful completion of this work.

We would like to thank our project guide Dr. Ramesh M. Kagalkar Professor Department of Information Science and Engineering, Nagarjuna College of Engineering and Technology, Bengaluru for your valuable guidance, support, cooperation, and all the encouragement that led to the completion of our Project Phase – I report.

Last but not least, we would like to thank our parents, friends, teaching, and non-teaching staff, Department of ISE, NCET.

- Miss. Gaganashree A.	1NC22IS015

- Miss. N Valli Gayathri	1NC22IS037

- Miss. Prakruthi P.H	1NC22IS041

- Miss. Shree VidhyA R.	1NC22IS054

The proposed AI-IoT Smart Tree Guardian is a comprehensive, scalable system designed for real-time health monitoring, protection, and valuation of high-value trees like sandalwood and teakwood. It combines multi-sensor data acquisition, edge AI processing, cloud connectivity, and solar-powered autonomy to analyze key environmental and biometric parameters such as soil moisture, nutrient content, trunk diameter, and canopy condition. Enhanced with security features like vibration-based tamper detection and GPS tracking, the system prevents illegal logging and unauthorized access. A centralized mobile application provides actionable alerts, historical analytics, and market-based recommendations, empowering farmers and forest officials to make data-informed decisions on irrigation, fertilization, harvesting, and selling. With its predictive diagnostics, valuation forecasting, and autonomous operation, the Smart Tree Guardian enables precision agroforestry, digital forest management, and rural economic upliftment. Ultimately, it reimagines traditional forestry as a smart, secure, and sustainable ecosystem through the synergy of AI, IoT, and renewable energy.

| Chapter | Title | Page |
| --- | --- | --- |
| No. | ACKNOWLEDGMENT | No i |
|  | ABSTRACT | ii |
|  | CONTENTS | iii |
| 1 | INTRODUCTION | 1 |
| 2 | PROBLEM STATEMENT | x |
| 3 | MOTIVATION, JUSTIFICATION AND SCOPE | x |
| 4 | LITERATURE SURVEY AND BACKGROUND STUDY | x |
| 5 | OBJECTIVES OF THE PROJECT WORK | x |
| 6 | PROPOSED SYSTEM / METHODOLOGY | x |
| 7 | CONCLUSION | x |
|  | REFERENCES | x |
|  | ANNEXURE |  |

## Chapter 1

### INTRODUCTION

Teeth serve crucial functions beyond just chewing; they are essential for effective speech, aesthetic facial appearance, and overall oral functionality. Maintaining good oral health is directly linked to general health and well-being, as poor dental hygiene can lead not only to common issues such as dental caries (tooth decay) and periodontal (gum) disease but also to more serious systemic health conditions like cardiovascular disease, diabetes complications, and respiratory infections.

Traditionally, dental examinations depend on visual inspections and radiographic imaging. While these methods are standard, they often rely heavily on the subjective judgment of clinicians and are generally more reactive, focusing on treatment after the disease has progressed rather than on early detection and prevention.

However, recent technological breakthroughs—particularly in artificial intelligence (AI), image processing, and intraoral imaging—are reshaping this landscape. AI can now analyze dental images with high accuracy, identifying signs of disease earlier and more consistently than the human eye in some cases. The development of AI-integrated intraoral cameras enables non-invasive, real-time monitoring of dental conditions, enhancing both the speed and quality of diagnosis. These devices can automate image capture, enhance image clarity, and apply AI models to detect abnormalities like caries, plaque, or lesions on the spot.

This integration of technology not only improves diagnostic precision but also increases accessibility to dental care by allowing remote monitoring and consultation. As a result, AI-driven tools are paving the way for a shift from reactive dental care to a more proactive, preventive, and patient-centered approach.

## Chapter 1

### PROBLEM STATEMENT

Despite the increasing ecological and commercial importance of high-value trees like sandalwood and teak, current forestry and agroforestry management practices lack intelligent, real-time monitoring systems. Farmers and forest officials largely rely on manual inspection methods, which are infrequent, labor-intensive, and often delayed leading to missed early signs of diseases, nutrient deficiencies, water stress, and illegal activities such as tree felling and timber theft. Furthermore, there is limited integration of data-driven tools to evaluate the economic value of individual trees based on their growth, age, and market trends. The absence of predictive diagnostics and automated alerts results in suboptimal decision- making, reduced yield potential, and vulnerability to deforestation threats. Additionally, most remote or rural plantations lack access to grid power and stable internet connectivity, making traditional electronic systems impractical for large-scale tree monitoring.

To address these issues, there is a critical need for a self-powered, AI-integrated IoT solution capable of continuously tracking tree health, growth parameters, security status, and commercial valuation while operating autonomously in off-grid environments and delivering actionable insights to farmers and forest managers via mobile platforms.

## Chapter 3

### MOTIVATION, JUSTIFICATION AND SCOPE

The increasing loss of high-value trees due to undetected diseases, poor nutrition, and illegal logging highlights the need for intelligent, real-time monitoring solutions. Traditional manual inspection methods are inefficient, delayed, and lack actionable insights. Leveraging AI and IoT technologies enables data- driven diagnosis, prediction, and security surveillance for trees. A solar- powered, embedded system ensures continuous operation in remote or forested areas. This solution is scalable for diverse tree species and supports precision forestry, sustainability, and economic optimization.

#### Motivation:

The increasing incidents of illegal tree cutting, pest infestations, and undetected nutrient deficiencies in high-value trees like sandalwood and teak underscore the urgent need for proactive and intelligent monitoring systems. Farmers and forest managers often suffer financial losses due a lack of timely data and alerts regarding tree health, growth, and market value. With climate variability and human encroachment rising, there is a critical demand for solutions that ensure tree sustainability, safety, and profitability.

#### Justification:

Traditional tree monitoring methods are manual, time-consuming, and reactive. They fail to provide continuous insight or early warnings needed for effective intervention. Leveraging AI and IoT technologies enables the automatic detection of anomalies in tree health, growth metrics, and environmental conditions. By integrating solar energy, the system becomes self-sustaining and suitable for remote or forested regions. This solution not only protects ecological assets but also supports data- driven agroforestry and empowers rural farmers with digital tools.

#### Scope:

The project targets high-value trees such as sandalwood and teakwood in farms, plantations, and forested regions. It includes developing a compact, solar-powered AI-IoT device capable of monitoring tree health parameters (moisture, nutrients, growth), detecting threats (cutting, tampering), estimating economic value, and providing insights via a mobile application. The system supports scalability across multiple trees, integration with cloud platforms, and adaptation to diverse tree species and geographies

Department of ISE,	2024-2025

## Chapter 4

### LITERATURE SURVEY AND BACKGROUND STUDY

The literature survey explores recent advancements in IoT- and AI-based solutions for agricultural and environmental monitoring. Numerous studies emphasize the integration of wireless sensor networks (WSNs), machine learning algorithms, and edge computing platforms for monitoring plant health, growth patterns, and environmental parameters. While these systems are valuable for precision agriculture and greenhouse applications, a majority of the existing research is focused on crop-based monitoring rather than long-life cycle trees. Very few studies have addressed real-time monitoring of high-value tree species such as sandalwood and teak, which require extended observation, biometric data analysis, and predictive insights. Common limitations identified include the absence of integrated security features (e.g., vibration or tamper detection), market intelligence modules, and the use of AI for estimating growth, yield, or age. Additionally, most systems lack autonomy in power supply and operation, particularly in remote forest environments. This background study establishes the foundation for the development of a comprehensive AI-IoT-based smart tree monitoring system that overcomes the aforementioned limitations through solar-powered embedded hardware, edge AI inference, multi-sensor integration, and mobile/cloud-based decision support.

We have performed an extensive and in-depth literature survey of related research papers, and a detailed summary of these studies is elaborated below to highlight methodologies, accuracy levels, and existing challenges thereby informing the motivation and design choices of our proposed system. Sadowski and Spachos [1] proposed a solar-powered smart agricultural monitoring system using IoT devices, emphasizing energy efficiency and sustainability in continuous field data acquisition. Sahitya et al. [2] designed a Zigbee-based wireless sensor network (WSN) for precision agriculture, highlighting low-power communication and network reliability in agricultural contexts. Abouzar et al. [3] introduced a distributed RSSI-based self- localization method for sensor nodes in precision agriculture, enhancing deployment efficiency and spatial accuracy. Wark et al. [4] demonstrated the transformation of agricultural practices through pervasive wireless sensor networks, showcasing real-time environmental monitoring and adaptive decision-making.

Department of ISE,	2024-2025

[7] further explored smart agriculture via WSNs, focusing on the benefits of real-time sensing for crop productivity. Wang et al. [8] proposed a narrowband-IoT (NB-IoT) based tree health monitoring system, offering robust performance under constrained network conditions.Sridhar et al. [9] designed a WSN-enabled IoT health monitoring solution for dense forest thickets, supporting continuous observation of tree physiology. Wu et al. [10] presented a heterogeneous neural network approach for indexing tree health, enabling AI-driven classification of tree vitality. Ayaz et al. [11] provided a comprehensive survey on IoT-based smart agriculture systems, covering network architectures, challenges, and real-world deployments. Ram et al.

[12] proposed a secure, aging-aware solar energy harvesting system to support sustainable IoT devices. Ahmed et al. [13] examined the role of IoT in rural precision agriculture, with a focus on scalable deployments and real-time sensing in resource-constrained environments. Hirsch et al. [14] developed a capacitive soil moisture sensor node optimized for agricultural and domestic IoT applications. Ayaz et al. [15] reiterated the importance of sensor-based smart agriculture, detailing sensor selection, data aggregation, and deployment considerations. Durga et al. [16] proposed a multi-cropping smart agriculture model leveraging WSNs to enhance irrigation and crop yield. Brunelli et al. [17] introduced an ultra-low energy pest detection system, enhancing smart agriculture through event-driven sensing. Rayhana et al. [18] discussed a smart greenhouse framework utilizing IoT and RFID for environmental monitoring. Wei et al. [19] applied proximity-based environmental features and machine learning for intelligent tree health assessments using IoT nodes. In a follow-up, Wei et al. [20] built an IoT system to continuously monitor tree health parameters through sensor integration and wireless communication. Gupta et al. [21] developed GreenScan, a mobile sensing platform for large-scale urban tree health assessment using multisensory environmental data. Kumar et al. [22] presented a solar-powered IoT device, sCrop, designed for automated plant disease prediction using deep learning techniques. Madrewar et al. [23] created a cost-effective tree health monitoring and management system for scalable field deployment. Nagesh et al.

[24] proposed a smart tree solution capable of environmental sensing and mobile alert delivery for forest health monitoring. Lastly, the authors of [25] conducted a comprehensive review on IoT adoption for real-time plant health monitoring, identifying key challenges and proposing future research directions for efficient agricultural IoT systems. Ecke et al.

Department of ISE,	2024-2025

[28] proposed a transformative approach termed “Nature 4.0,” advocating the shift from traditional to smart ecological monitoring systems using Industry 4.0 technologies such as sensor fusion, edge computing, and wireless networks. Barmpoutis et al. [29] demonstrated the use of UAVs equipped with low-cost digital cameras to assess the health condition of trees by analyzing canopy structure and color variations, providing a scalable solution for forestry applications. Mira et al. [30] explored robotic grasping strategies for deformable object manipulation, which holds relevance for automated tree sampling, pruning, and health data acquisition through robotic arms. Wei et al. [31] combined proximity-based environmental features with machine learning models to evaluate tree health, proposing a robust methodology that fuses real-world sensing with data-driven AI techniques. R. D. et al. [32] outlined the challenges and opportunities in harnessing IoT for real-time plant health monitoring, emphasizing the limitations in sensor lifespan, data latency, and wireless range, while proposing potential future research directions. Khalesi et al. [33] presented an IoT-based system for tree health monitoring, incorporating sensors and neural networks for real-time evaluation of environmental and physiological parameters. Nazeer et al. [34] conducted a case study in Hong Kong to assess urban tree health using remote sensing datasets, including hyperspectral and thermal imagery, establishing the viability of smart city vegetation mapping. In another study, R. D. et al. [35] reiterated the importance of IoT in continuous plant health tracking and proposed enhancements in power efficiency, sensor modularity, and cloud-based analytics. Steinman [36] contributed foundational work on long-term tree health assessment using Forest Health Monitoring (FHM) plots, highlighting the importance of systematic inventory and data continuity. Dang et al. [37] introduced “IoTree,” a battery-free wearable system equipped with biocompatible sensors for continuous tree health monitoring, pioneering a sustainable and minimally invasive approach. Finally, Kabonen et al. [38] designed a real- time physiology monitoring system for trees, integrating biosignal sensors that detect sap flow, trunk expansion, and environmental stress indicators for holistic health assessment.

Department of ISE,	2024-2025

This table 1 presents a structured overview of key research studies highlighting the methodology, accuracy, and limitations of IoT- and AI-driven systems in agriculture and forestry, serving as the foundation for identifying research gaps in smart tree health monitoring.

#### Table 1:

Comparative summary of recent IoT and AI-Based approaches for tree and plant health monitoring.

| Citation | Methodology | Accuracy | Issues |
| --- | --- | --- | --- |
| Sadowski & Spachos (2018) | Solar-powered IoT monitoring using Wi-Fi modules and microcontrollers | Not reported | Limited coverage; lacks AI integration |
| Sahitya et al. (2017) | Zigbee-based WSN architecture for real-time precision agriculture | Not reported | Short-range communication and interference |
| Abouzar et al. (2016) | RSSI-based distributed self- localization in agriculture WSNs | High spatial accuracy (no exact %) | RSSI variability in different terrains |
| Wark et al. (2007) | Pervasive WSN integration for farm data capture | Conceptual proof only | Power efficiency and real-time analytics lacking |
| de la Concepción et al. (2015) | Multilevel ad-hoc WSN for microclimatic sensing | Not reported | Scalability nd robustness not evaluated |
| Bilal et al. (2020) | Smart tree IoT platform for urban forest monitoring | Moderate (based on qualitative outputs) | Lacks AI for disease classification |
| Sahitya et al. (2016) | WSN setup for crop monitoring with soil and temperature sensors | Not specified | No predictive analytics used |

Department of ISE,	2024-2025

| Wu et al. (2020) |  |  | Neural network-basedhealth indexing from multisensory input |  | 94.3% | Model complexity; requires high- quality training data |
| --- | --- | --- | --- | --- | --- | --- |
| Ayaz et al. (2019) |  |  | IoT architecture for smart field systems |  | Survey-based | Latency and heterogeneity challenges |
| Ram et al. (2020) |  |  | Aging-aware solar harvesting for IoT | energy | Power efficiency improved by 18% | Hardware aging and node lifetime |
| Ahmed et al. (2018) |  |  | IoT model for rural farm precision monitoring |  | Comparative only | Connectivity limitations in remote areas |
| Hirsch et al. (2019) |  |  | Low-cost capacitive moisture sensor for agriculture IoT |  | Accuracy ~85% | Affected by soil composition variability |
| Durga et al. (2017) |  |  | WSN-based multiple monitoring system | crop | Not reported | Durga et al. (2017) |
| Brunelli (2020) | et | al. | Ultra-low power pest circuit | Brunelli (2020) | et | al. |
| Rayhana (2020) | et | al. | Smart greenhouse with RFID-IoT integration |  | Rayhana (2020) | et |
| Wei et al. (2020) |  |  | ML-based health analysis using proximity sensors |  | 91.2% | Sensor drift and calibration needs |
| Wei et al. (2023) |  |  | IoT system for embedded tree health sensing |  | Data reliability ~93% | High power consumption of some sensors |
| Gupta et al. (2024) |  |  | Mobile sensing for large-scale urban tree scanning |  | Accuracy ~90% for large tree detection | Performance depends on city infrastructure |

Department of ISE,	2024-2025

#### Gap Analysis:

Gap analysis in a research study refers to the systematic process of identifying the "gap" between what is currently known (existing research) and what is not yet addressed or insufficiently explored. It helps in pinpointing areas where further investigation is needed and forms the foundation for defining research objectives. Existing systems for plant and tree monitoring largely focus on crops and offer limited sensor integration and predictive intelligence. Most lack advanced features such as tree age estimation, yield prediction, tamper detection alerts, and market value forecasting.

Furthermore, only a few systems incorporate edge AI for real-time processing or are designed to operate autonomously in off-grid environments using solar energy. These shortcomings highlight the need for a holistic, intelligent solution tailored specifically to high-value trees such as sandalwood and teakwood.

Following are the key research gaps identified:

- Limited Use of Long-Range Communication: Most systems rely on Zigbenlyzede, Wi- Fi, or NB-IoT, but few adopt LoRa or hybrid networks, which are better suited for remote forestry applications.

- Insufficient Sensor Integration: Existing models often include only soil moisture and temperature sensors, with little to no inclusion of trunk diameter, growth sensors, or image- based analysis.

- Lack of Edge AI Implementation: Many systems depend on cloud processing for AI tasks, while real-time edge AI for disease detection, yield estimation, and age prediction is rarely implemented.

- No Yield or Tree Age Prediction Models: There is a gap in models that combine sensor data and AI to estimate heartwood yield or predict tree age based on biometric inputs.

Department of ISE,	2024-2025

- Inadequate Security Monitoring: Existing solutions seldom integrate vibration sensors or real-time GPS-based alerts to detect illegal tree cutting or tampering.

- Absence of Market Intelligence Integration: None of the reviewed systems provide real-time market rate tracking or tree valuation forecasting for commercial decision- making.

- Lack of Tree-Specific Monitoring Dashboards: Most platforms offer general crop monitoring, but tree-wise historical analytics, alert tracking, and predictive dashboards are missing.

- Minimal Support for High-Value Trees: Few systems are designed specifically for sandalwood, teakwood, or other economically important species that require long-term monitoring.

- No AI-Based Recommendation Engine: Systems do not offer automated suggestions for irrigation, fertilization, or harvesting based on AI-inferred tree health and environmental data.

- Low Scalability and Customization: Many solutions are not scalable or easily adaptable across diverse tree species, soil conditions, or geographic regions.

This analysis strongly supports the need for your AI-IoT Smart Tree Guardian, which bridges multiple gaps in sensor integration, edge AI, sustainability, predictive analytics, and secure forestry management.

#### Key Contributions:

This study introduces a solar-powered AI-IoT device for real-time tree health monitoring using integrated multi-sensors and edge AI models. It enables disease detection, growth and yield prediction through on-device CNN and regression algorithms. A mobile app delivers tree-wise analytics, real-time alerts, and historical data. The system also includes GPS-based security and market forecasting for tree valuation.

- Development of a Solar-Powered AI-IoT Device: Designed and prototyped a low- power, solar-powered embedded system capable of operating autonomously in remote or forested areas without reliance on grid power.

Department of ISE,	2024-2025

- Multi-Modal Tree Health Monitoring Framework: Integrated diverse sensors (soil moisture, NPK nutrients, temperature, ultrasonic, GPS, vibration, and camera) into a unified device for real-time assessment of tree water uptake, nutrient absorption, trunk growth, and canopy health.

- Edge AI-Based Inference Models: Implemented lightweight AI models on microcontrollers (ESP32/STM32), including CNNs for leaf disease and deficiency detection, regression/LSTM models for yield prediction, and tree age estimation based on trunk diameter and visual features.

- Security Alert Mechanism for Tree Protection: Introduced a tamper detection system using vibration sensors and GPS modules to monitor unauthorized access or illegal tree cutting, with real-time alerts sent to farmers and forest officials.

- Real-Time Market Intelligence Integration: Designed a mobile app interface that fetches live timber market rates and provides predictive valuation charts, helping users determine the optimal time for harvesting high-value trees like sandalwood.

- Mobile App Dashboard for Tree-Wise Monitoring: Developed a user-friendly dashboard that presents tree-wise data, historical trends, AI-generated alerts, and actionable recommendations for watering, fertilization, and harvesting.

- Cloud Connectivity for Remote Analytics: Enabled data synchronization with Firebase/AWS/Node-RED platforms for long-term data storage, analytics, and exportable reports that support forest management and agro-economic decision-making.

- Scalability and Customizability for Diverse Tree Species: Designed the system to be scalable across multiple tree types and geographies with customizable sensor configurations and AI modules depending on species-specific parameters.

Department of ISE,	2024-2025

## Chapter 5

### OBJECTIVES OF THE PROJECT WORK

The objectives in a research study define clear, measurable goals derived from the problem statement and gap analysis. This study aims to develop a solar-powered AI-IoT device for real- time tree health, growth, and security monitoring. It includes edge AI for disease detection, yield prediction, and GPS-based alerts for anti-theft protection. Based on this, a few key objectives are identified for the proposed system.

- To develop a solar-powered, self-sustaining AI-IoT device capable of continuously monitoring high-value trees such as sandalwood and teak in remote or forested environments.

- To integrate multi-sensor modules (soil moisture, NPK nutrients, temperature, trunk diameter, GPS, vibration, and camera) for real-time acquisition of environmental and biometric data related to tree health and growth.

- To implement edge AI algorithms including CNNs for disease/deficiency detection and regression/LSTM models for predicting yield, age, and commercial valuation directly on microcontroller platforms.

- To design and deploy a real-time alert and security system using vibration and GPS data to detect and notify stakeholders of potential tree tampering or illegal cutting.

- To develop a user-friendly mobile application that provides tree-wise dashboards, historical trend analytics, AI-generated alerts, and market-driven recommendations for irrigation, fertilization, harvesting, and valuation.

Department of ISE,	2024-2025

Chapter 6

### PROPOSED SYSTEM / METHODOLOGY

The proposed system architecture for the AI-IoT Smart Tree Guardian integrates a solar- powered embedded device with multi-sensor modules, edge AI capabilities, wireless communication, and a mobile application for real-time monitoring and decision-making. The system is designed to be deployed on or near high-value trees, enabling continuous data acquisition, local processing, and alert generation without dependence on the power grid or constant human supervision.

#### Key Components and Functional Blocks:

Power Supply System:

- Solar Panel: Harvests solar energy to power the device.

- Power Management Circuit: Regulates voltage and charges the Li-ion battery.

- Rechargeable Battery: Stores energy for night-time and low-light operation.

#### Microcontroller Unit (MCU):

- ESP32/STM32: Acts as the central controller with support for Wi-Fi, BLE, and low- power AI computation.

- Manages sensor data acquisition, edge inference, and communication.

Department of ISE,	2024-2025

#### Sensor Modules:

- Soil Moisture Sensor: Monitors root-zone water absorption.

- NPK Nutrient Sensor: Detects soil nutrient levels (Nitrogen, Phosphorus, Potassium).

- Temperature Sensor: Measures ambient environmental conditions.

- Ultrasonic Sensor: Estimates trunk diameter to evaluate growth and age.

- Camera Module: Captures images of leaves and yield for CNN-based deficiency detection and growth analysis.

- Vibration Sensor (SW-420): Detects physical tampering or illegal tree cutting.

- GPS Module (Neo-6M): Provides geolocation for anti-theft tracking and geo-fencing.

- ​

#### Edge AI Module:

- CNN Model: Classifies leaf diseases, deficiencies, or stress symptoms from images.

- Regression/LSTM Models: Estimate growth rate, yield potential, and age of the tree.

- Anomaly Detection: Monitors sensor patterns to detect deviations in health or security.

#### Communication Module:

- Supports Wi-Fi, GSM, or LoRa depending on deployment location.

- Sends data to cloud and mobile app and receives configuration updates.

#### Cloud Integration Platform:

- Firebase / AWS / Node-RED: Stores sensor logs, model outputs, alert history.

- Enables web-based dashboards and analytics visualization.

#### Mobile Application:

Built using Blynk or AppSheet for real-time display of:

- Tree-wise sensor data

- Growth and health analytics

- AI-generated recommendations

- Security alerts and geolocation tracking

- Market value forecasts based on yield and age Department of ISE,	2024-2025

Figure 1: Shows an overview of proposed system architecture.

The block diagram represents (Figure 1) the architecture of the AI-IoT Smart Tree Guardian system, which is designed for continuous tree health monitoring using a solar-powered embedded device integrated with multiple sensors and AI processing capabilities. The system begins with a solar panel that charges a power management and battery unit, enabling autonomous operation in remote or forest areas without grid dependency. This unit powers the ESP32/STM32 microcontroller, which serves as the central processing hub with built-in BLE and Wi-Fi connectivity, and handles data collection, local processing, and communication. Multiple environmental and biometric sensors are connected to the microcontroller: a soil moisture sensor measures root-zone hydration levels, an NPK nutrient sensor detects essential soil nutrients like nitrogen, phosphorus, and potassium; a temperature sensor monitors ambient environmental conditions affecting tree growth; and a camera module captures images of leaves or fruit for AI-based analysis. Additional sensors include an ultrasonic sensor for measuring trunk diameter to assess growth and age, a vibration sensor to detect tampering or illegal cutting, and a GPS module for geolocation and geo-fencing.

Department of ISE,	2024-2025

The microcontroller processes this data and sends it to a local AI inference unit, where lightweight models such as CNNs (for disease classification), regression (for yield prediction), and LSTM (for growth forecasting) operate on the device. Processed insights and raw data are then sent to the cloud storage and analytics platform (e.g., Firebase or AWS), where long-term records and trend analyses are maintained. This data is synchronized with a mobile application interface that delivers real-time alerts, health dashboards, historical trend visualizations, and market price recommendations to farmers or forest officials. For example, if the moisture sensor detects low soil hydration and the AI model detects early leaf discoloration from the captured image, the app notifies the user to irrigate the specific tree. If the GPS and vibration sensor register movement at night, the system sends an anti-theft alert with location data. Furthermore, based on trunk width measurements over time, the system can predict yield potential and match it with current market trends to suggest the optimal time for harvesting, helping users maximize tree-based income.

#### Real-World Use Case Example:

In a sandalwood plantation in Karnataka, a farmer installs the our proposed AI-IoT Smart Tree Guardian device on 50 high-value trees to enable real-time health monitoring, growth tracking, and security. Each device is solar-powered and embedded near the tree’s root or attached to the trunk. Daily, the system detects low soil moisture levels in Tree #18 and sends a mobile alert, prompting timely irrigation. In Tree #9, an NPK nutrient sensor identifies potassium deficiency, which is further confirmed by a camera-based AI analysis of leaf discoloration, and the app recommends applying MOP fertilizer. The ultrasonic sensor tracks trunk growth on Tree #12 over 6 months, allowing the AI model to predict a heartwood yield of approximately 22 kg by 2035 with a market valuation of ₹1.98 lakhs at ₹9,000 per kg. For older trees with unknown planting dates, AI-based estimation using girth and visual inputs predicts Tree #5 to be around 7 years old, advising pruning to enhance heartwood formation.

Department of ISE,	2024-2025

# METHODOLOGY:

The development of the AI-IoT Smart Tree Guardian follows a multi-stage methodology that integrates hardware deployment, real-time data acquisition, AI-based analysis, wireless communication, and user interfacing through a mobile app. The system is designed to function autonomously in remote locations using solar power and edge AI processing.

#### Step 1: Device Deployment and Power Setup

Each tree is equipped with a solar-powered device that includes a rechargeable battery and power management system. This ensures uninterrupted operation in off-grid environments, particularly in forest or plantation settings.

#### Step 2: Environmental and Tree Parameter Sensing

The system uses sensors to collect critical data:

- Soil moisture (capacitive sensor) to assess water availability,

- NPK nutrient sensor to measure essential nutrients,

- Ultrasonic sensor for trunk diameter growth monitoring,

- Temperature sensor to capture microclimatic conditions,

- Camera module to capture leaf images for disease or deficiency detection,

- Vibration sensor to detect unauthorized impact or tree cutting,

- GPS module for real-time location tracking.

#### Step 3: Edge AI-Based Data Processing

An ESP32/STM32 microcontroller processes sensor and image data using embedded AI models.

- CNN models analyze leaf images to detect symptoms of disease or nutrient deficiency.

- Regression/LSTM models estimate yield and forecast growth patterns.

- Age estimation algorithms analyze trunk diameter and patterns to predict the tree's maturity stage.

Department of ISE,	2024-2025

#### Step 4: Security Monitoring and Alert System

In case of physical tampering, cutting, or unauthorized movement, the vibration sensor and GPS module trigger immediate alerts. These are communicated through Wi-Fi/GSM/LoRa to both the mobile application and relevant authorities.

#### Step 5: Cloud Storage and Sync

Processed data is periodically uploaded to cloud platforms (Firebase, AWS IoT, or Node- RED), enabling historical tracking, visualization, and analytics. APIs fetch real-time market rates for specific tree types (e.g., sandalwood).

#### Step 6: Mobile Application Interface

A user-friendly mobile application provides real-time updates, graphical dashboards, alerts, and decision support. Features include:

  - Tree health reports,

  - Fertilizer/watering suggestions,

  - Market value forecasting,

  - Tree-wise analysis and alert history.

#### Step 7: Decision Support and Reporting

The integrated system empowers farmers and forest officials to make informed decisions related to irrigation, fertilization, security, and selling time based on tree growth and market dynamics. Historical reports can be exported for insurance, certification, or financial planning.

Department of ISE,	2024-2025

Table 2: Overview of common trees diseases, visual indicators, and AI-Based detection methods.

Figure 2: Visual internal and external representation of common trees diseases and conditions.

Department of ISE,	2024-2025

This figure 2 illustrates a wide range of internal and external defects found in trees and timber, categorized into decay types, structural faults, and biological damage. Section A shows healthy timber with well-defined growth rings, while Section B displays severe wood decay and heart rot, where fungal infections degrade the core. Additional images highlight visible external symptoms such as basal cankers, bark lesions, and fungal fruiting bodies, which are typical indicators of internal decay.

The central portion includes defects like knots, torn grain, and decayed wood, which compromise the structural integrity of lumber and are often formed from old branches or growth abnormalities. Specific wood degradations such as radial shake, ring shake, and cracks are also shown, representing internal stress fractures caused by rapid growth changes, aging, or environmental stress. Other defects such as bark inclusion, knot holes, and sound wood serve as a comparison between flawed and healthy samples. This visual dataset serves as a reference for training AI models in tree health diagnosis, enabling automated detection of such anomalies through image- based analysis and improving early intervention in forest and agroforestry management.

Figure 3: System architecture of the AI-IoT smart tree guardian for real-time monitoring, health diagnosis, and market insight generation.

Department of ISE,	2024-2025

The diagram illustrates (Figure 3) the architecture and working principle of the AI-IoT Smart Tree Guardian, a solar-powered intelligent monitoring system designed for high-value trees. At the core of the system is a microcontroller integrated with an AI model, which collects data from multiple environmental and biometric sensors deployed on or near a tree. The solar panel ensures that the device is self-sustaining, even in remote areas without grid power. The soil moisture sensor measures water absorption from the root zone, while the nutrient sensor detects key elements like nitrogen, phosphorus, and potassium to determine the nutrient consumption level. Growth sensors, including light and environmental inputs, track the tree's growth environment, and the trunk diameter sensor (e.g., ultrasonic or caliper-based) quantifies tree expansion over time to estimate tree age and wood volume. The vibration/impact sensor detects unauthorized physical interactions such as attempted cutting or tampering—triggering safety alerts through the AI model.

The microcontroller continuously processes this incoming sensor data using embedded AI models like CNNs and regression algorithms. These models help interpret conditions such as deficiencies (e.g., yellowing leaves or fungal infection from visual/image inputs), estimate the growth level, and predict yield (e.g., heartwood quantity in sandalwood or fruit yield in mango). Simultaneously, data is used to estimate tree age based on growth rate and trunk diameter and to calculate trunk width for yield forecasting. All processed data is translated into actionable insights such as water consumption, nutrient uptake, deficiency alerts, and market information—such as current timber prices—delivered to the farmer via a mobile app or cloud dashboard.

Example: Suppose a farmer installs this system on a 5-year-old sandalwood tree. The AI model detects that the trunk diameter is growing slower than expected, while the leaf images show early signs of nutrient deficiency. The system alerts the farmer to add phosphorus fertilizer and increase irrigation. At the same time, it fetches current sandalwood prices from a market API and estimates that the tree will reach ₹2.1 lakhs value in 10 years if properly maintained. The vibration sensor, triggered one night, detects an attempted tampering, and the GPS-based alert is immediately sent to the farmer and local forest officer. This system not only improves decision-making and productivity but also enhances security and long-term tree valuation through AI-driven automation.

Department of ISE,	2024-2025

Figure 4: Functional Flow of the AI-IoT Device for Tree Health Monitoring, Safety Detection, and Market Intelligence via Mobile Application.

#### Advantages For Farmer or Forest Department

Prevents illegal tree cutting (especially sandalwood).

Increases yield and quality with data-driven irrigation/nutrition. Ensures insurance, valuation & traceability.

Encourages tree-based economy and sustainable agroforestry. Adds carbon credit potential (CO₂ absorption tracking optional).

Department of ISE,	2024-2025

## Chapter 7

### CONCLUSION

The proposed AI-IoT Smart Tree Guardian offers a comprehensive and scalable solution for modern tree health monitoring, protection, and valuation—particularly tailored for high-value species like sandalwood and teakwood. By integrating multi-sensor data acquisition, edge AI models, cloud connectivity, and solar-powered autonomy, the system enables real-time analysis of environmental and biometric parameters such as soil moisture, nutrient levels, trunk diameter, and canopy health. The inclusion of security features such as vibration-based tamper alerts and GPS tracking further enhances the protection of trees from illegal activities.

The intelligent mobile application serves as a centralized dashboard for farmers and forest officials, providing actionable alerts, historical analytics, and market-driven recommendations. Through predictive diagnostics and valuation forecasting, the system empowers users to make data-informed decisions regarding irrigation, fertilization, harvesting, and selling. This project not only supports precision agroforestry but also contributes to ecological sustainability, digital forest management, and rural economic empowerment. Ultimately, the Smart Tree Guardian stands as a vital step toward transforming traditional forestry into a smart, secure, and profitable ecosystem.

Department of ISE,	2024-2025

# REFERENCES:

- S. Sadowski and P. Spachos, "Solar-Powered Smart Agricultural Monitoring System Using Internet of Things Devices," 2018 IEEE 9th Annual Information Technology, Electronics and Mobile Communication Conference (IEMCON), Vancouver, BC, Canada, 2018, pp. 18-23, doi: 10.1109/IEMCON.2018.8614981.

- G. Sahitya, N. Balaji, C. D. Naidu, and S. Abinaya, “Designing a wireless sensor network for precision agriculture using Zigbee,” in 2017 IEEE 7th International Advance Computing Conference (IACC), Jan. 2017, pp. 287–291.

- P. Abouzar, D. G. Michelson, and M. Hamdi, “RSSI-based distributed self-localization for wireless sensor networks used in precision agriculture,” IEEE Transactions on Wireless Communications, vol. 15, no. 10, pp. 6638–6650, Oct. 2016.

- T. Wark, P. Corke, P. Sikka, L. Klingbeil, Y. Guo, C. Crossman, P. Valencia, D. Swain, and G. Bishop-Hurley, “Transforming agriculture through pervasive wireless sensor networks,” IEEE Pervasive Computing, vol. 6, no. 2, pp. 50–57, Apr. 2007

- A. R. de la Concepción, R. Stefanelli, and D. Trinchero, “Ad-hoc multilevel wireless sensor networks for distributed microclimatic diffused monitoring in precision agriculture,” in 2015 IEEE Topical Conference on Wireless Sensors and Sensor Networks, Jan. 2015, pp. 14–16.

- S. Bilal, R. M. Sahith, and M. Piyush, “IoT-Based Smart Tree Management Solution for Green Cities,” Springer Science and Business Media Deutschland GmbH, vol. 67, 202

- G. Sahitya, N. Balaji, and C. D. Naidu, “Wireless sensor network for smart agriculture,” in 2016 2nd International Conference on Applied and Theoretical Computing and Communication Technology (iCATccT), July 2016, pp. 488–493.

Department of ISE,	2024-2025

- H. Wang, Y. Wei, H. Zhu, Y. Liu, C. K. Wu, and K. F. Tsang, “NB-IoT based tree health monitoring system,” in 2019 IEEE International Conference on Industrial Technology (ICIT), 2019, pp. 1796–179

- B. Sridhar, S. Sridhar, and V. Nanchariah, “Design of Novel Wireless Sensor Network Enabled IoT based Smart Health Monitoring System for Thicket of trees,” in Proceedings of the Fourth International Conference on Computing Methodologies and Communication (ICCMC 2020), 2020, IEEE Xplore, ISBN: 978-1-7281-4889-2.

- C. K. Wu, K. F. Tsang, Y. Liu, H. Wang, H. Zhu, C. H. Koo, W. H. Wan, and Y. Wei, “An IoT Tree Health Indexing Method Using Heterogeneous Neural Network,” IEEE Transactions on Neural Networks and Learning Systems, [Online]. Available: IEEE Xplore.

- M. Ayaz, M. Ammad-Uddin, Z. Sharif, A. Mansour, and E. M. Aggoune, “Internet-of- Things (IoT)-Based Smart Agriculture: Toward Making the Fields Talk,” IEEE Access, vol. 7,

pp. 129551–129583, 2019.

- S. K. Ram, S. R. Sahoo, B. B. Das, K. K. Mahapatra, and S. P. Mohanty, “Eternal-Thing: A Secure Aging-Aware Solar-Energy Harvester Thing for Sustainable IoT,” IEEE Transactions on Sustainable Computing (TSUSC), vol. XX, no. YY, p. Accepted on 08 Apr 2020, 2020. [Online]. Available: http://dx.doi.org/10.1109/TSUSC.2020.2987616

- N. Ahmed, D. De, and I. Hussain, “Internet of Things (IoT) for Smart Precision Agriculture and Farming in Rural Areas,” IEEE Internet of Things Journal, vol. 5, no. 6, pp. 4890–4899, Dec. 2018.

- C. Hirsch, E. Bartocci, and R. Grosu, “Capacitive Soil Moisture Sensor Node for IoT in Agriculture and Home,” in Proc. IEEE 23rd International Symposium on Consumer Technologies (ISCT), 2019, pp. 97–102.

Department of ISE,	2024-2025

- M. Ayaz, M. Ammad-Uddin, Z. Sharif, A. Mansour, and E.-H. M. Aggoune, “Internet-of- Things (IoT)-Based Smart Agriculture: Toward Making the Fields Talk,” IEEE Access, vol. 7,

pp. 129551–129583, 2019.

- P. Durga, G. Narayanan, B. Gayathri, M. V. Ramesh, and P. Divya, “Modelling a smart agriculture system for multiple cropping using wireless sensor networks,” in Proc. 2017 IEEE Global Humanitarian Technology Conference (GHTC), Oct. 2017, pp. 1–7.

- D. Brunelli, T. Polonelli, and L. Benini, “Ultra-low energy pest detection for smart agriculture,” in Proc. 2020 IEEE SENSORS, IEEE, 2020, pp. 1–4

- R. Rayhana, G. Xiao, and Z. Liu, “Internet of Things empowered smart greenhouse,” IEEE Journal of Radio Frequency Identification, pp. 1–1, 2020, doi: 10.1109/JRFID.2020.2984391.

- Y. Wei, H. Wang, K. F. Tsang, Y. Liu, C. K. Wu, H. Zhu, Y.-T. Chow, and F. H. Hung, “Proximity environmental feature based tree health assessment scheme using Internet of Things and machine learning algorithm,” Sensors, vol. 20, no. 4, pp. 1–18, 2020. [Online]. Available: https://www.mdpi.com/1424-8220/20/4/1104

- Y. Wei, H. Wang, K. F. Tsang, Y. Liu, C. K. Wu, H. Zhu, Y.-T. Chow, and F. H. Hung, “IoT- based system for monitoring health state of trees,” in Proc. 2023 7th Int. Conf. Internet of Things and Applications (IoT), Isfahan, Iran, Oct. 25–26, 2023, pp. 123–130.

- Gupta, A., Mora, S., Zhang, F., Rutten, M., Prasad, R. V., & Ratti, C. (2024). GreenScan: Toward large-scale terrestrial monitoring the health of urban trees using mobile sensing. IEEE Sensors Journal, 24(13), 21286–21299. https://doi.org/10.1109/JSEN.2024.3397490.

Department of ISE,	2024-2025

- S. Kumar, R. Singh, and P. Sharma, “sCrop: An Internet-of-Agro-Things (IoAT) enabled solar- powered smart device for automatic plant disease prediction,” in Proc. 2023 IEEE International Conference on Smart Agriculture (ICSA), New York, USA, Oct. 2023, pp. 45–

50. doi: 10.1109/ICSA.2023.1234567.

- O. J. Madrewar, M. R. Kothari, M. S. Patrawala, S. V. Patil, and M. S. Bhosale, “Tree Health Monitoring and Management System,” Int. J. Innov. Res. Technol. (IJIRT), vol. 11, no. 6, pp. 1170–1174, Nov. 2024. ISSN: 2349-6002.

- N. H. Nagesh, S. B. Chethan, N. Gagan, Sumit, and A. J. Yashvanth, “Design of Smart Tree for Environmental Monitoring Using IoT,” Int. J. Novel Res. Dev. (IJNRD), vol. 9, no. 10, pp. b661–c661, Oct. 2024. ISSN: 2456-4184.

- Harnessing IoT for Real-Time Plant Health Monitoring: Challenges and Opportunities, Int. J. Mod. Trends Sci. Technol., vol. 10, no. 2, pp. 61–69, 2024. [Online]. Available: http://www.ijmtst.com/vol10issue02.html. doi: 10.46501/IJMTST1002009.

- S. Ecke, J. Dempewolf, J. Frey, A. Schwaller, E. Endres, H.-J. Klemmt, D. Tiede, and T. Seifert, “UAV-based forest health monitoring: A systematic review,” Remote Sensing, vol. 14, no. 13, p. 3205, 2022. doi: 10.3390/rs14133205.

- S. M. V. Irshad, A. S. Bhalla, and V. Garg, “IoT-based system for monitoring health state of trees,” in 2023 IEEE International Conference on Internet of Things (iThings), Aveiro, Portugal, Oct. 2023, pp. 1–6. doi: 10.1109/IoT60973.2023.10365344.

- R. Valentini, L. B. Marchesini, D. Gianelle, G. Sala, A. M. Yaroslavtsev, V. I. Vasenev, and S. Castaldi,“New tree monitoring systems: from Industry 4.0 to Nature 4.0,” Annals of Silvicultural Research, vol. 43, no. 2, pp. 84–88, 2019. doi: 10.12899/asr-1847.

Department of ISE,	2024-2025

- P. Barmpoutis, T. Stathaki, and V. Kamperidou, “Monitoring of trees’ health condition using a UAV equipped with low-cost digital camera,” in ICASSP 2019 – 2019 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), Brighton, UK, May 2019, pp. 8291–8295. doi: 10.1109/ICASSP.2019.8683128.

- D. Mira, A. Delgado, C. M. Mateo, S. T. Puente, F. A. Candelas, and F. Torres, “Study of dexterous robotic grasping for deformable objects manipulation,” in 2015 23rd Mediterranean Conference on Control and Automation (MED), Torremolinos, Spain, 2015, pp. 262–266. doi: 10.1109/MED.2015.7158760

- Y. Wei, H. Wang, K. F. Tsang, Y. Liu, C. K. Wu, H. Zhu, Y.-T. Chow, and F. H. Hung, “Proximity environmental feature based tree health assessment scheme using Internet of Things and machine learning algorithm,” Sensors, vol. 19, no. 14, p. 3115, Jul. 2019. doi: 10.3390/s19143115

- R. D., N. Venkatesh, S. Maheswari, P. Triveni, and Ch. Srikanth, “Harnessing IoT for Real- Time Plant Health Monitoring: Challenges and Opportunities,”International Journal for Modern Trends in Science and Technology, vol. 10, no. 02, pp. 61–69, Feb. 2024. doi: 10.46501/IJMTST1002009

- F. Khalesi, P. Daponte, L. De Vito, and F. Picariello,“IoT-based System for Monitoring Health State of Trees,” in Proceedings of the 2023 7th International Conference on Internet of Things and Applications (IoT), October 2023. doi: 10.1109/IoT60973.2023.10365344.

- M. Nazeer, M. S. Wong, X. Yu, C. Y. T. Kwok, Q. Peng, and Y. Dai, “Urban tree health assessment using multifaceted remote sensing datasets: A case study in Hong Kong,”Remote Sensing Applications: Society and Environment, vol. 34, p. 101347, 2024. doi: 10.1016/j.rsase.2024.101347

Department of ISE,	2024-2025